SEARCH PATTERNS     SMC generally dictates the pattern

track spacing (max 1 nm)  [person 600 ft] = sweep width (1/2 the distance each side of vessel)

Track line   run parallel to estimated track of the lost vessel  (with return)

½ the sweep width offset (down current side)    then return on the other side of the estimated track

Expanding Square   All right 900 turns

Leg 1 & 2  from datum  ½ the sweep width  other pairs of legs X2 ~ 900 turns to right

Second pattern ~ rotate 450

Sector search    three equilateral triangles

1st  leg  length = sweep width then  1200 degree right turns to complete equilateral triangle

Then continue straight onto the 1st leg of the 2nd triangle likewise for the 3rd triangle

Second pattern ~ rotate 300

Parallel Track  (start at ½ the sweep width in form each side at a corner) 1st leg: run parallel to the      MAJOR  AXIS

Make a 900 turn and run the sweep width distance then make a 900 turn (back tracking) etc.

Creeping line   (start at ½ the sweep width in form each side at a corner) 1st leg: run perpendicular to the MAJOR  AXIS

|  |  |  |  | Knots |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| speeds: | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 12 | 15 | 20 |
| Distance | run time | run time | run time | run time | run time | run time | run time | run time | run time | run time | run time |
| 0.1 | 2.0 | 1.5 | 1.2 | 1.0 | 0.9 | 0.8 | 0.7 | 0.6 | 0.5 | 0.4 | 0.3 |
| 0.2 | 4.0 | 3.0 | 2.4 | 2.0 | 1.7 | 1.5 | 1.3 | 1.2 | 1.0 | 0.8 | 0.6 |
| 0.3 | 6.0 | 4.5 | 3.6 | 3.0 | 2.6 | 2.3 | 2.0 | 1.8 | 1.5 | 1.2 | 0.9 |
| 0.4 | 8.0 | 6.0 | 4.8 | 4.0 | 3.4 | 3.0 | 2.7 | 2.4 | 2.0 | 1.6 | 1.2 |
| 0.5 | 10.0 | 7.5 | 6.0 | 5.0 | 4.3 | 3.8 | 3.3 | 3.0 | 2.5 | 2.0 | 1.5 |
| 0.6 | 12.0 | 9.0 | 7.2 | 6.0 | 5.1 | 4.5 | 4.0 | 3.6 | 3.0 | 2.4 | 1.8 |
| 0.7 | 14.0 | 10.5 | 8.4 | 7.0 | 6.0 | 5.3 | 4.7 | 4.2 | 3.5 | 2.8 | 2.1 |
| 0.8 | 16.0 | 12.0 | 9.6 | 8.0 | 6.9 | 6.0 | 5.3 | 4.8 | 4.0 | 3.2 | 2.4 |
| 0.9 | 18.0 | 13.5 | 10.8 | 9.0 | 7.7 | 6.8 | 6.0 | 5.4 | 4.5 | 3.6 | 2.7 |
| 1.0 | 20.0 | 15.0 | 12.0 | 10.0 | 8.6 | 7.5 | 6.7 | 6.0 | 5.0 | 4.0 | 3.0 |
| 1.1 | 22.0 | 16.5 | 13.2 | 11.0 | 9.4 | 8.3 | 7.3 | 6.6 | 5.5 | 4.4 | 3.3 |
| 1.2 | 24.0 | 18.0 | 14.4 | 12.0 | 10.3 | 9.0 | 8.0 | 7.2 | 6.0 | 4.8 | 3.6 |
| 1.3 | 26.0 | 19.5 | 15.6 | 13.0 | 11.1 | 9.8 | 8.7 | 7.8 | 6.5 | 5.2 | 3.9 |
| 1.4 | 28.0 | 21.0 | 16.8 | 14.0 | 12.0 | 10.5 | 9.3 | 8.4 | 7.0 | 5.6 | 4.2 |
| 1.5 | 30.0 | 22.5 | 18.0 | 15.0 | 12.9 | 11.3 | 10.0 | 9.0 | 7.5 | 6.0 | 4.5 |
| NM |  |  |  | MINUTES |  |  |  |  |  |  |  |